==============================================================================

MAKETCP.DOC for maketcp 1.1d                              VERSION: 13-Apr-1995

==============================================================================

Table of Contents

=================

1    Introduction

2    Running maketcp

3    TCP files

4    Creating a maketcp input file

5    History

5.1    Version 1.1c (27-Nov-1992)

5.2    Version 1.1d (13-Apr-1995)

1 Introduction

==============

maketcp generates (or decodes) TCP files (TeX code page).  These files

are used by emTeX (and TeXconv) for code page conversion as well as

to convert special input characters into TeX commands.  By using TCP

files, emTeX can be adapted to different code pages.  Note that your

texts may not be usable with other TeX implementations if you're using

TCP files.  TeXconv can be used for converting your input files for

use with other TeX implementations.

maketcp can also generate a .tex file for mapping input characters to

TeX commands.

2 Running maketcp

=================

The maketcp command line has the following syntax:

maketcp <mode> [<options>] [<input_file> [<output_file>]]

The following choices are available for <mode>:

-c      Convert a text file into a TCP file (generate a TCP file;

compile).  Using a TCP file is recommended for code page

conversion as code page conversion cannot be completely

implemented with TeX commands.

-d      Convert a TCP file into a text file (decompile).

-t      Convert a text file into a .tex file.  Using a .tex file is

recommended for mapping input characters to TeX commands as

this will also work with other TeX implementations and with

appropriate verbatim commands.

The following options are available:

-8      Set up a conversion table which leaves the characters with

codes 128 to 255 unchanged (without -8, the codes from 128 to

255 are mapped to code 127)

-r      Set up a conversion table which leaves the characters with

codes 0 to 31 unchanged (without -r, codes 0 to 31 are mapped

to code 127)

-x      Also write commands for code page conversion to the .tex file;

by default, only commands for mapping input characters to TeX

commands are written.  The -x option is used for mode -t only

The default extension for TCP files is .tcp (with the option -c this

applies to the output file, with -d to the input file).

3 TCP files

===========

A TCP file contains three tables: the first shows which special

characters are to be converted into TeX control sequences; the second

converts the characters in the input file into the internal TeX

characters and the third performs the reverse of this.

When no TCP file is used with emTeX, only the characters with codes

0, 9, 12 and 32 to 126 are permitted.  When the /r option is given,

characters in the range 0 through 31 are also allowed.  With the /8

option (only with INITEX) characters in the range 128 through 255 are

permitted as well.  No conversion takes place.

This behaviour can be altered by using a TCP file.  Characters in the

range 128 through 255 can be converted into TeX control sequences; in

addition, the conversion of all characters for input and output can

be changed -- this is necessary when TeX's character coding does not

correspond to the code page in use.  This should only be done for the

characters above 128 as otherwise unexpected effects will be caused.

Conversion for output will only affect the LOG file, not the DVI file.

4 Creating a maketcp input file

===============================

To create a TCP file you must make a text file in which both the

translation of special characters into TeX control sequences and the

character conversion for input and output is given.  You can get an

example of the format of the file by converting the TCP file supplied

(850_tex.tcp) into the equivalent text file with the command

maketcp -d 850_tex.tcp example.txt

The text file can contain comments which are lines with a `%' in the

first column; you can also make your file more readable by inserting

blank lines, which are ignored.  All other lines are either special

character conversions or input to internal character conversions --

the output character conversion table is constructed from the input

table.  Characters to be converted can be entered either as is (a

single character) or in hexadecimal, in the TeX 3.0 format (^^ff).

The translation of a special character is entered as follows:

^^84 -> \"a     % Umlaut-a

The line begins with the special character (Umlaut-a), followed by a

space, an arrow (hyphen and greater than character), space and then

the TeX control sequence which is to replace the character.  The `%'

and the text following it up to the end of the line will be ignored

unless it is part of the TeX command -- in the following, ONE space

will not be ignored:

^^fe -> \%\     % Tex control sequence: "\%\ "

The conversion of an input character into an internal code (and an

internal code into an output character) is entered as follows:

^^84 ^^e4               % Umlaut-a (PC) -> Umlaut-a (ISO 8859/1)

The line begins with the input character followed by a space and the

desired internal representation (as coded in the TFM file).  When the

character ^^e4 is to be output, it will be converted into ^^84 (in

this example).  If more than one character is mapped into the same

internal character then the last conversion will be used for output:

a b

b b

In this pointless (but simple) example, both a and b are mapped on to

the same internal character: by the rule above, b will be converted

into b on output.  This feature is important if you want to map

extended characters to ASCII characters.  You should use

^^af z

z z

to translate ^^af to z and to make z display z.  If you are using only

^^af z

then ^^: will be displayed for z.  (Moreover, a z in a file name would

be converted into ^^: causing grief when opening the file!)

The default conversion tables are changed by such a text file: these

conversion tables are set up by the -8 and -r options (see above) --

in some circumstances, you may save yourself some work by using the -8

and/or -r options.  When you convert a TCP file into a text file, the

output shows only the differences from the default tables set up by

the -8 and -r options: the text file output will therefore depend on

the settings of these options.

5 History

=========

5.1 Version 1.1c (27-Nov-1992)

------------------------------

- Bug fixed.  That bug caused emTeX displaying a character entered in

^^ff format to be displayed as character without conversion if there

is no conversion from an external character to that character.  Now,

those characters are displayed in ^^ff format.

- Support of HPFS filenames under OS/2.

5.2 Version 1.1d (13-Apr-1995)

------------------------------

- The new -t option lets maketcp write a .tex file which maps external

character codes to TeX commands.

-------- End of MAKETCP.DOC -------------